# Advanced Video Analytics

#### Lab5-1: Chaining models: Use multiple models in an application

- Set environmental variables

$ source /opt/intel/computer_vision_sdk/bin/setupvars.sh

$ export SV=/opt/intel/workshop/smart-video-workshop

$ cd $SV/object-detection

- Run the executable for the security barrier sample with the mobilenet-ssd* model used in the first tutorial

$ ./lab.py security_barrier_camera_sample -i car_1.bmp -m mobilenet-ssd -d CPU

InferenceEngine:

API version ............ 1.0

Build .................. 10073

[ INFO ] Parsing input parameters

[ INFO ] No extensions provided

[ INFO ] Reading input

[ INFO ] Loading plugin CPU

API version ............ 1.0

Build .................. lnx_20180314

Description ....... MKLDNNPlugin

[ INFO ] Loading network files for VehicleDetection

[ INFO ] Batch size is forced to  1

[ INFO ] Checking Vehicle Detection inputs

[ INFO ] Checking Vehicle Detection outputs

[ INFO ] Loading Vehicle Detection model to the CPU plugin

[ INFO ] Vehicle Attribs detection DISABLED

[ INFO ] License Plate Recognition detection DISABLED

[ INFO ] Start inference

[ INFO ] Execution successful

- Run the security camera sample with ICV models

$ ./lab.py security_barrier_camera_sample -d CPU -i car_1.bmp -m vehicle-license-plate-detection-barrier-0007 -m_va vehicle-attributes-recognition-barrier-0039 -m_lpr license-plate-recognition-barrier-0001

InferenceEngine:

API version ............ 1.0

Build .................. 10073

[ INFO ] Parsing input parameters

[ INFO ] No extensions provided

[ INFO ] Reading input

[ INFO ] Loading plugin CPU

API version ............ 1.0

Build .................. lnx_20180314

Description ....... MKLDNNPlugin

[ INFO ] Loading network files for VehicleDetection

[ INFO ] Batch size is forced to  1

[ INFO ] Checking Vehicle Detection inputs

[ INFO ] Checking Vehicle Detection outputs

[ INFO ] Loading Vehicle Detection model to the CPU plugin

[ INFO ] Loading network files for VehicleAttribs

[ INFO ] Batch size is forced to 1 for Vehicle Attribs

[ INFO ] Checking VehicleAttribs inputs

[ INFO ] Checking Vehicle Attribs outputs

[ INFO ] Loading Vehicle Attribs model to the CPU plugin

[ INFO ] Loading network files for Licence Plate Recognition (LPR)

[ INFO ] Batch size is forced to  1 for LPR Network

[ INFO ] Checking LPR Network inputs

[ INFO ] Checking LPR Network outputs

[ INFO ] Loading LPR model to the CPU plugin

[ INFO ] Start inference

[ INFO ] Execution successful

#### Lab5-2: Use multiple models on different hardware

- The face detection sample from the OpenVINO™ tookit package

$ ./lab.py interactive_face_detection_sample -h

InferenceEngine:

API version ............ 1.0

Build .................. 10073

interactive_face_detection [OPTION]

Options:

-h                         Print a usage message.

-i "<path>"                Optional. Path to an video file. Default value is "cam" to work with camera.

-m "<path>"                Required. Path to an .xml file with a trained face detection model.

-m_ag "<path>"             Optional. Path to an .xml file with a trained age gender model.

-m_hp "<path>"             Optional. Path to an .xml file with a trained head pose model.

-l "<absolute_path>"     Required for MKLDNN (CPU)-targeted custom layers.Absolute path to a shared library with the kernels impl.

Or

-c "<absolute_path>"     Required for clDNN (GPU)-targeted custom kernels.Absolute path to the xml file with the kernels desc.

-d "<device>"              Specify the target device for Face Detection (CPU, GPU, FPGA, or MYRYAD. Sample will look for a suitable plugin for device specified.

-d_ag "<device>"           Specify the target device for Age Gender Detection (CPU, GPU, FPGA, or MYRYAD. Sample will look for a suitable plugin for device specified.

-d_hp "<device>"           Specify the target device for Head Pose Detection (CPU, GPU, FPGA, or MYRYAD. Sample will look for a suitable plugin for device specified.

-n_ag "<num>"              Specify number of maximum simultaneously processed faces for Age Gender Detection ( default is 16).

-n_hp "<num>"              Specify number of maximum simultaneously processed faces for Head Pose Detection ( default is 16).

-no_wait                   No wait for key press in the end.

-pc                        Enables per-layer performance report.

-r                         Inference results as raw values.

-t                         Probability threshold for detections.

- Check if a web cam is connected

$ ls /dev/video*

/dev/video0

- Run the face demo, face detection only, on MYRIAD

$ ./lab.py interactive_face_detection_sample -i /dev/video0 -m FP16/face-detection-retail-0004 -d MYRIAD

InferenceEngine:

API version ............ 1.1

Build .................. 12419

[ INFO ] Parsing input parameters

[ INFO ] Reading input

[ INFO ] Loading plugin MYRIAD

API version ............ 1.1

Build .................. 12419

Description ....... myriadPlugin

[ INFO ] Loading network files for Face Detection

[ INFO ] Batch size is set to  1

[ INFO ] Checking Face Detection inputs

[ INFO ] Checking Face Detection outputs

[ INFO ] Loading Face Detection model to the MYRIAD plugin

[ INFO ] Age Gender DISABLED

[ INFO ] Head Pose DISABLED

[ INFO ] Emotions Recognition DISABLED

[ INFO ] Start inference

[ INFO ] Press any key to stop

- Now we add (to the face detection) also an age and gender detection, running on the CPU

$ ./lab.py interactive_face_detection_sample -i /dev/video0 -m FP16/face-detection-retail-0004 -d MYRIAD -m_ag age-gender-recognition-retail-0013 -d_ag CPU

InferenceEngine:

API version ............ 1.1

Build .................. 12419

[ INFO ] Parsing input parameters

[ INFO ] Reading input

[ INFO ] Loading plugin MYRIAD

API version ............ 1.1

Build .................. 12419

Description ....... myriadPlugin

[ INFO ] Loading plugin CPU

API version ............ 1.1

Build .................. lnx_20180510

Description ....... MKLDNNPlugin

[ INFO ] Loading network files for Face Detection

[ INFO ] Batch size is set to  1

[ INFO ] Checking Face Detection inputs

[ INFO ] Checking Face Detection outputs

[ INFO ] Loading Face Detection model to the MYRIAD plugin

[ INFO ] Loading network files for AgeGender

[ INFO ] Batch size is set to 16 for Age Gender

[ INFO ] Checking Age Gender inputs

[ INFO ] Checking Age Gender outputs

[ INFO ] Age layer: age_conv3

[ INFO ] Gender layer: prob

[ INFO ] Loading Age Gender model to the CPU plugin

[ INFO ] Head Pose DISABLED

[ INFO ] Emotions Recognition DISABLED

[ INFO ] Start inference

[ INFO ] Press any key to stop

- Now let’s add head position detection running on GPU.

$ ./lab.py interactive_face_detection_sample -i /dev/video0 -m FP16/face-detection-retail-0004 -d MYRIAD -m_ag age-gender-recognition-retail-0013 -d_ag CPU -m_hp head-pose-estimation-adas-0001 -d_hp GPU

InferenceEngine:

API version ............ 1.1

Build .................. 12419

[ INFO ] Parsing input parameters

[ INFO ] Reading input

[ INFO ] Loading plugin MYRIAD

API version ............ 1.1

Build .................. 12419

Description ....... myriadPlugin

[ INFO ] Loading plugin CPU

API version ............ 1.1

Build .................. lnx_20180510

Description ....... MKLDNNPlugin

[ INFO ] Loading plugin GPU

API version ............ 1.1

Build .................. ci-main-03703

Description ....... clDNNPlugin

[ INFO ] Loading network files for Face Detection

[ INFO ] Batch size is set to  1

[ INFO ] Checking Face Detection inputs

[ INFO ] Checking Face Detection outputs

[ INFO ] Loading Face Detection model to the MYRIAD plugin

[ INFO ] Loading network files for AgeGender

[ INFO ] Batch size is set to 16 for Age Gender

[ INFO ] Checking Age Gender inputs

[ INFO ] Checking Age Gender outputs

[ INFO ] Age layer: age_conv3

[ INFO ] Gender layer: prob

[ INFO ] Loading Age Gender model to the CPU plugin

[ INFO ] Loading network files for Head Pose detection

[ INFO ] Batch size is sey to  16 for Head Pose Network

[ INFO ] Checking Head Pose Network inputs

[ INFO ] Checking Head Pose network outputs

[ INFO ] Loading Head Pose model to the GPU plugin

[ INFO ] Emotions Recognition DISABLED

[ INFO ] Start inference

[ INFO ] Press any key to stop